GUIDE SPECIFICATION

All Glass Entrances

Coral Architectural Products 3010 Rice Mine Road Tuscaloosa, AL. 35406

Voice: (800) 772-7737

Fax: (800) 443-6261

SECTION 08450 ALUMINUM ENTRANCES AND STOREFRONTS

This suggested guide specification has been developed using the current edition of the Construction Specifications Institute (CSI) “Manual of Practice,” includ- ing the recommendations for the CSI 3 Part Section Format and the CSI Page Format. Additionally, the development concept and organizational arrangement of the American Institute of Architects (AIA) MASTERSPEC Program was recognized in the preparation of this guide specification. Neither CSI nor AIA endorse specific manufacturers and products. The preparation of the guide specification assumes the use of standard contract documents and forms, including the “Conditions of the Contract,” published by the AIA.

PART 1 – GENERAL

  - Summary

    - Section Includes: All Glass Vestibule Doors matching entrance doors and sidelites by Coral Architectural Products.

      - Types of Coral Architectural Products All Glass Doors;

        - .[Type “A”] Swing Door; Fully Tempered Heavy Glass Door, Partial Rails or Patch Fittings at top and bottom pivot

corners (locks are not available in Type “A” door) glass thickness range from 3/8”, ½” and ¾”.

        - .[Type “F”] Swing Door; Fully Tempered Heavy Glass Door, Partial Rails or Patch Fittings at top and bottom pivot

corners and bottom lock, glass thickness range from 3/8”, ½” and ¾”.

        - .[Type “P”] Swing Door; Fully Tempered Heavy Glass Door, Full Width Top and Bottom Rails, glass thickness

range from 3/8”, ½” and ¾”.

d. [Type “AP”] Swing Door; Fully Tempered Heavy Glass Door, Top Patch Fitting and Full Width Bottom Rail, glass

thickness range from 3/8”, ½” and ¾”.

    - Related Sections:

      - Section 08450 – All Glass Entrances

      - Section 08491 – Sliding Doors

      - Section 08700 – Finish Hardware* Lock cylinders for tempered glass entrance doors are specified in Division 8

Section “Finish Hardware”

  - Submittals

    - Product Data: Submit Manufacturer’s product data for all glass entrance systems including:

      - Standard details and fabrication method.

      - Data on product finish, hardware and accessories.

      - Recommendations for maintenance and cleaning of exterior finish surfaces.

      - Test data on fabricated entrance system.

    - Shop Drawings for each type of all glass entrance system are required, including;

      - Layout and installation details.

      - Elevations.

      - Detail sections of fittings

      - Hardware mounting heights.

      - Anchorage and reinforcement.

      - Glazing details.

    - Samples for approval;

      - Submit pairs of samples of each specified metal color and finish on 9-inch long sections of extrusions or formed

Shapes.

      - Submit samples of glass approximately 6 inches square indicating the edge conditions.

GUIDE SPECIFICATION

All Glass Entrances

  - Warranty

    - Project Warranty: Refer to “Conditions of the Contract” for project warranty provisions.

    - Manufacturer’s Product Warranty: Submit, for Owner’s acceptance, manufacturer’s warranty for entrance system as follows:

      - Warranty Period: Two (2) years from Date of Substantial Completion of the project provided however that the Limited Warranty shall

begin in no event later than six months from the initial date of shipment by Coral Architectural Products.

  - Quality Assurance

    - Qualifications:

      - Installer Qualifications: Engage an experienced installer who has completed installations of all glass entrances that are similar in

design and extent to those required for the project and whose work has resulted in construction with a record of successful

service performance.

      - Manufacturer Qualifications: Provide all glass entrances furnished by a firm experienced in manufacturing all glass entrance

systems that are similar to those indicated for this project and that have a record of successful service performance.

(All door rail systems must be tested)

      - Single Source Responsibility: Obtain all glass entrance systems from a single manufacturer, to ensure full compatibility and

warranty of parts.

      - Design criteria: The drawings indicate the size, profile and dimensional requirements of the all glass entrance system required

and are based on the specific types and models indicated. All glass entrances by other manufacturers may be considered,

provided deviations in dimensions and profiles are minor and do not change the design concept as judged by the Architect.

The burden of proof of equality is on the proposer.

      - Safety glass standard: Provide tempered glass components that comply with ANSI Z97.1 and testing requirements of CPSC

16 CFR  1201  Category II.

      - Testing criteria for Door Rail: The door rail must be tested to perform 1,000,000 cycles without any failures. The door rail

should also be subject to a temperature pull-off test at temperatures from -10ºF to 150ºF (-23ºC to 65.5ºC).  The rail shall remain

stationary throughout this test while a 500 pound (227 kg) pressure is applied.

    - Pre-Installation Meetings: Conduct pre-installation meeting to verify project requirements, substrate conditions, manufacturer’s instal lation instructions, and manufacturer’s warranty requirements.

  - Delivery, Storage, and Handling

    - Deliver all glass entrances and related components in the manufacturer’s original protective packaging. Do not deliver entrance

units until the work is ready for their installation.

      - Inspect components for damage upon delivery. Unless minor defects in metal components can be made to meet

the Architect’s specifications and satisfaction, damaged parts should be removed and replaced.

  - Project Conditions

    - Field Measurements: Check opening by accurate field measurement before fabrication. Show recorder measurements on shop

drawings. Coordinate fabrication schedule with construction progress to avoid delay of the work and possible damage to the

finished product.

      - Where necessary, proceed with fabrication without measurement and coordinate fabrication tolerances to ensure

proper fit.

PART 2 – PRODUCTS

GUIDE SPECIFICATION

All Glass Entrances

  - Manufacturers (Acceptable Manufacturers/Products)

    - Basis of design: Design is based on an “All-Glass” Entrance Door System featuring heavy tempered glass secured to a CRL Wedge-Lock®

Door Rail System, manufactured by:

C.R. Laurence Co., Inc. (CRL)

Tel: (800) 421-6144 Fax: (800) 587-7501

Email: architectural@crlaurence.com

www.crlaurence.com

fabricated by:

Coral Architectural Products, A Division of Coral Industries

Tel: (800) 772-7737 Fax: (800) 443-6261

Website: www.coralap.com

    - Subject to compliance with requirements, “all glass” entrances from other manufacturers meeting the specified requirements may be acceptable.

  - Materials

    - Glass: Provide flat, fully tempered glass in thickness indicated for doors and sidelites. Comply with requirements of

ASTM C 1048 for FT (fully tempered), Condition A (uncoated surfaces), Type 1 (transparent) Class 1 (clear) glass. Provide

products of thickness indicated that have been tested for surface and edge compression according to ASTM C 1048 and

for impact strength according to 16 CFR Part 1201 for Category II materials.

      - Thickness: 3/8 inch (10 mm)

      - Thickness: 1/2 inch (12 mm)

      - Thickness: 5/8 inch (16 mm)

      - Thickness: 3/4 inch (19 mm)

      - Edge treatment: Provide machine ground and polished edges for exposed glass edges of doors and sidelites and

flat ground edges for butting glass edges.

      - Glass Manufacturers:	.

    - Fittings, General: Provide CRL Wedge-Lock® Door Rails in required profile, size and glass thickness as selected by the Architect.

Comply with requirements indicated for kind and form of metal finish.

      - Aluminum: Provide fittings fabricated from aluminum extrusions of alloy and temper recommended by manufacturer

for use intended and required for application of finish indicated, but not less than strength and durability properties

specified in ASTM B 221 for 6063-T5.

    - Door Rail Systems: Provide door rail systems matching metal and finish of door fittings. The system shall include, but not limited to, door rails, patches, vertical stiles, center locks, and strike housings. Comply with GANA guidelines, and hardware manufacturer requirements for size restrictions. Door rails shall allow for jamb adjustment in or out with standard hardware. System shall include, but not limited to, end caps, blocking and preparation.

      - Profile: Square

      - Profile: Beveled (Only available in 4 inch [102 mm] high profile)

      - Height: 2-5/16 inch (59 mm)

      - Height: 4 inch (102 mm)

      - Height: 6 inch (152 mm)

      - Height: 10 inch (254 mm)

      - Height: Custom as indicated on drawings

    - Accessory Fittings: Provide manufacturer’s standard accessory fittings of the type indicated. Comply with requirements indicated for kind and form of metal and finish of door fittings.

    - Anchors and Fasteners: Manufacturer’s standard concealed anchors and fastenings. Do not use exposed fasteners.

    - Weatherstripping: Can be applied to edges of glass and top/bottom door rails to help reduce air and water infiltration.

The weatherstripping shall be pile and replaceable without removing doors from opening.

S3

  - Hardware

GUIDE SPECIFICATION

All Glass Entrances

    - General: Provide heavy-duty hardware units as indicated, scheduled or required for operation of each type of door,

including the following items of sizes, numbers and type recommended by the manufacturer for the type of service

required. Provide metal and finish for exposed parts to match the finish of the door rails.

    - CRL by C.R. Laurence Co., 9100 Series Medium Duty extended spindle Overhead Concealed Door Closers and 9200 Series Heavy-Duty extended spindle Overhead Concealed Door Closers are double acting and fit into CRL 4-1/2” x 1-3/4” Single or Double Closer Headers. Closers secure to the center hung arm in the top door rail. Provide top adjustable arm, bottom adjustable pivot and finished cover plate if required. Comply with manufacturer’s recommendations for closer size, depending upon door size, exposure to weather, and anticipated frequency of use.

    - C.R. Laurence Co., J990 Series Heavy Weight Floor Mounted Closers are double acting and supplied with cement box and Brushed S/S cover plate. Cover plates in other finishes are available. Provide bottom adjustable arm, top adjustable pivot and finished cover plate if required. Comply with manufacturer’s recommendations for closer size, depending upon door size, exposure to weather, and anticipated frequency of use.

Include the following:

      - Non-hold-open closers must comply with ADA Handicap requirements.

      - Consult manufacturer for closer recommendations on doors over 264 lbs.

    - Push-Pull Set: Provide handles selected by the Architect and supplied by C.R. Laurence.

    - Concealed PA100 and PA110 Panic Handles: Panic device shall be 1-1/4” diameter C.R. Laurence’s PA100 or PA110 with interior operating panic handle in combination with exterior fixed pull handles designated by letters. Panic

mechanism shall be concealed within the brass or stainless steel tubing. Entrance from exterior by a keyed cylinder is optional.

    - Deadbolt Lock Handles: Shall be C.R. Laurence’s DB100, DB110, DB130, DB140, DB150, DB160, DB170 with interior fixed handle and any of the fixed exterior pull handles designated by letters. Operating mechanism shall be

concealed within the 1-1/4” brass or stainless steel tubing. There shall be a keyed cylinder on both sides or a thumbturn on the interior side where indicated. The locking mechanism shall be on the interior side of the door.

    - Electronic Egress Control Handles: Shall be C.R. Laurence’s EG100 or EG110 with interior operating egress handle in combination with exterior fixed pull handles designated by letters. Panic mechanism shall be concealed within the brass or stainless steel tubing. Entrance from exterior by a keyed is cylinder optional.

    - Electric Strikes: Shall be Folger Adams 310-1 with ¾” straight latch bolt keeper without signal switches using PA 100 Panic Handles. Electric strikes are mounted in the header or transom bar.

    - Locks: Equip exterior doors with manufacturer’s locksets that accept a standard cylinder with related components. Comply with the following:

      - Location and function: Provide round throw deadbolt in continuous bottom fitting. Lock to be operated by key outside and thumbturn inside with end load capability.

    - Cylinders or Magnetic Locks: Supplied as described under Division 8 section, for keying into building system.

    - Threshold: Provide manufacturer’s standard extruded aluminum threshold in mill finish. Coordinate cutouts with operating hardware. Include anchors and jamb clips.

  - Fabrication

GUIDE SPECIFICATION

All Glass Entrances

    - General: Fabricate all glass entrance components to designs and sizes indicated. Size of door and profile requirements

of fittings and hardware are indicated on the drawings.

      - Locate and provide holes and cutouts in glass to receive hardware before tempering glass. Do not permit cutting,

drilling or other alterations to glass after tempering.

      - Fabricate work to accommodate required fittings, hardware, anchors, reinforcement, and accessory items.

    - Prefabrication: Complete fabrication, assembly, finishing, hardware application and other work to the greatest extent possible before shipment to the project site. Disassemble components only as necessary for shipment and installation.

    - Continuity: Maintain accurate relation of planes and angles with hairline fit of contracting members.

  - Metal Finishes (List one or more finishes)

    - US-28 Clear Satin Anodized Aluminum

313 Dark Bronze/Black Anodized

Black Powder Coat

US-32 Polished Stainless Steel Clad

US-32 D Brushed Stainless Steel Clad

US-10BNL Oil Rubbed Bronze Clad

US-3 Polished Brass Clad

PVD “Perma Brass” Polished Clad

Other Custom Metal and Painted finishes available

    - Preferable use of PVD “Perma Brass” over US-3 Polished Brass Cladding to reduce maintenance cost in the future.

PART 3 – EXECUTION

  - Examination

    - Examine substrates and supports with the installer, present for compliance with requirements indicated, installation

tolerances and other conditions that affect the installation of all glass entrances and storefronts. Correct unsatisfactory

conditions before proceeding with the installation.

      - Do not proceed with installation until unsatisfactory conditions are corrected.

  - Installation

    - Install all glass entrance door and associated components in accordance with manufacturer’s printed instructions and

recommendations.

      - Verify units level, plumb and true line.

      - Lubricate hardware and other moving parts.

  - Adjusting

    - Adjust doors and hardware to provide a tight fit at meeting points and at weather-stripping for smooth operation and weather

tight closure.

    - Hardware: Adjust operating hardware to ensure proper operation. Set, seal, and grout floor closer cases. Coordinate cylinder

installation.

  - Cleaning and Protection

    - Clean door and frame surfaces after installation, exercising care to avoid damage to the finish.

    - Clean glass surfaces after installation, complying with requirements contained in the “Glass and Glazing” section for cleaning and maintenance. Remove excess glazing sealant compounds, dirt or other substances.

    - Institute protective measures required throughout the remainder of the construction period to ensure that the all glass entrances do not incur any damage or deterioration, other than normal weathering, at the time of acceptance.

DISCLAIMER STATEMENT

This guide specification is to only be used by qualified construction specifiers. The guide specification is not intended to be verbatim as a project specification

without appropriate modifications for the specific use intended. The guide specification must be used and coordinated with the procedures of each design firm,

S5

and the particular requirements of a specific construction project.