January 01, 2011

To:	architect

firm

address

city, state, zip

From: Mike Rogers

re: 	project

Side-by-Side Product Comparisons

The table below attempts to compare Coral Architectural Products’ curtain wall and flush glaze systems with other specified manufacturers typically listed in project specifications. The performance information was derived from technical data publicly listed on each manufacturers website as of this date. This information is subject to change and should be verified via each manufacturers actual product test reports. Coral makes no representation as to the accuracy of performance information other than our own.

Manufacturer & System

Water

Structural

NFRC 102-2004

2-1/2" Curtain Wall

PSF

Allowed

(cfm)

Actual

(cfm)

U Factor

CRF

U Factor

Coral PW251™

6.24

0.060

0.01

20.00 PSF

No Leaks

+65 PSF

-60 PSF

0.43*

66

0.41

Kawneer 1600™

6.24

0.060

<0.060

12.00 PSF

No Leaks

40 PSF

0.43*

71

Vistawall Reliance

6.24

0.060

<0.060

15.00 PSF

No Leaks

60 PSF

0.63**

66

US Aluminum 3150

6.24

0.060

<0.060

15.00 PSF

No Leaks

25 PSF

0.37**

66

Manufacturer & System

Water

Structural

NFRC 102-2010

2" x 4-1/2" Center

Glazed Thermal

Storefront

PSF

Allowed

(cfm)

Actual

(cfm)

U Factor

CRF

U Factor

Coral FL300T™

6.24

0.060

0.01

15.00 PSF

No Leaks

60 PSF

0.40*

57

0.39

Kawneer 451T™

6.24

0.060

<0.060

8.00 PSF No

Leaks

20 PSF

0.44*

62

Vistawall FG-3000T

6.24

0.060

<0.060

10.00 PSF

No Leaks

37.5 PSF

0.63**

58

US Aluminum IT451

6.24

0.060

<0.060

10.00 PSF

No Leaks

35 PSF

0.60**

61

*with 1" Low-E / Clear insulating glass

**with 1" Clear / Clear insulating glass

Test data for

NFRC method

could not be

found on website

Test data for

NFRC method

could not be

found on website

ASTM E283

Thermal

AAMA 1503-98

Thermal

AAMA 1503-98

ASTM E331

ASTM E330

ASTM E330

ASTM E331

Air

ASTM E283

Air

Note: PW-251 and FL300 are also approved for use in the State of Florida (FPA FL8379.1 – 8379.4, FL8832.2-R1).